Integer Tokens

Integer Blocks

Double TokenHeight

Double BlockHeight

Function main

Motor On

Power High

Speed 30

Accel 30, 30

SpeedS 1300

AccelS 5000

Tool 1

Tokens = 2

Blocks = 2

TokenHeight = 6.0

BlockHeight = 6.0

Integer TokenID

Integer BlockID

Go Retract_Safe

' Definir sistemas locales

Local 1, P_Token_Origin, P_Token_X, P_Token_Y

Local 2, P_Block_Origin, P_Block_X, P_Block_Y

Local 3, P_Tray_Origin, P_Tray_X, P_Tray_Y

' Definir patrones de paletizado

Pallet 1, XY(0,0,0,0)/1, XY(0,0,TokenHeight,0)/1, XY(0,0,TokenHeight*2,0)/1, Tokens+1, 1

Pallet 2, XY(0,0,0,0)/2, XY(0,0,BlockHeight,0)/2, XY(0,0,BlockHeight*2,0)/2, Blocks+1, 1

Pallet 3, XY(0,0,0,0)/3, XY(30,0,0,0)/3, XY(60,0,0,0)/3, 1, Tokens+Blocks

For TokenID = Tokens To 0 Step -1

Pick_Infeed_Token()

Alignment_Token()

Place_Tray_Token()

Next TokenID

For BlockID = Blocks To 0 Step -1

Pick_Infeed_Block()

Alignment_Block()

Place_Tray_Block()

Next BlockID

Go Retract_Safe

Fend

Function Pick_Infeed_Token

Print "Picking Token from Pallet. Token ID = ", TokenID

Go PalletPos(1, TokenID, 0)/1 +Z(50) CP

Move PalletPos(1, TokenID, 0)/1

On 8

Wait .5

Move PalletPos(1, TokenID, 0)/1 +Z(50) CP

Fend

Function Pick_Infeed_Block

Print "Picking Block from Pallet. Block ID = ", BlockID

Go PalletPos(2, BlockID, 0)/2 +Z(50) CP

Move PalletPos(2, BlockID, 0)/2

On 8

Wait .5

Move PalletPos(2, BlockID, 0)/2 +Z(50) CP

Fend

Function Alignment_Token

Print "Aligning Token. Token ID = ", TokenID

Go Align_Token +Z(20) CP

Move Align_Token

Off 8

Move Align_Token

Move Align_Token +Z(5) CP

Go Align_Token +Z(5) CP

Move Align_Token

On 8

Wait .5

Move Align_Token +Z(20) CP

Fend

Function Alignment_Block

Print "Aligning Block. Block ID = ", BlockID

Go Align_Block +Z(20) CP

Move Align_Block

Off 8

Move Align_Block +Y(-5)

Move Align_Block +X(5) +Y(-6) CP

Move Align_Block +X(5) +Y(-6) +Z(5) CP

Go Align_Block +Z(5) CP

Move Align_Block

On 8

Wait .5

Move Align_Block +Z(20) CP

Fend

Function Place_Tray_Token

Print "Placing Token in Tray. Tray Position ID = ", TokenID

Go PalletPos(3, 0, TokenID)/3 +Z(20) CP

Move PalletPos(3, 0, TokenID)/3

Off 8

Wait .5

Move PalletPos(3, 0, TokenID)/3 +Z(50) CP

Fend

Function Place_Tray_Block

Print "Placing Block in Tray. Tray Position ID = ", Blocks + BlockID

Go PalletPos(3, 0, Blocks + BlockID)/3 +Z(20) CP

Move PalletPos(3, 0, Blocks + BlockID)/3

Off 8

Wait .5

Move PalletPos(3, 0, Blocks + BlockID)/3 +Z(50) CP

Fend

APARTE

Example

' Definir sistemas locales para cada pieza

Local 1, P1_1, P2_1, P3_1  ' Pieza 1

Local 2, P1_2, P2_2, P3_2  ' Pieza 2

Local 3, P1_3, P2_3, P3_3  ' Pieza 3

' Definir patrones de paletizado relativos a cada sistema local

PL1_1 = XY(0, 0, 0, 0)/1

PL2_1 = XY(100, 0, 0, 0)/1

PL3_1 = XY(0, 100, 0, 0)/1

Pallet 1, PL1_1, PL2_1, PL3_1, 4, 3

PL1_2 = XY(0, 0, 0, 0)/2

PL2_2 = XY(100, 0, 0, 0)/2

PL3_2 = XY(0, 100, 0, 0)/2

Pallet 2, PL1_2, PL2_2, PL3_2, 4, 3

PL1_3 = XY(0, 0, 0, 0)/3

PL2_3 = XY(100, 0, 0, 0)/3

PL3_3 = XY(0, 100, 0, 0)/3

Pallet 3, PL1_3, PL2_3, PL3_3, 4, 3

FOR r = 0 TO 3

FOR c = 0 TO 2

' Pieza 1

Jump PalletPos(1, r, c)/1

' Acción sobre pieza 1

' Pieza 2

Jump PalletPos(2, r, c)/2

' Acción sobre pieza 2

' Pieza 3

Jump PalletPos(3, r, c)/3

' Acción sobre pieza 3

NEXT

NEXT